22.

Effective Address = Address in Instruction + Value in Register R1

Effective Address = 0x500 + 0x200

Effective Address = 0x700

Value in AC = 0x800

25.

A.

There is a Read After Write data hazard. The second instruction depends on the result of the first instruction. The second instruction will try to fetch the value of x in its S3 stage. However, the first instruction will not have completed its S4 stage by that time. This means the second instruction will read the old incorrect value of x

B.

There is Read After Write data hazard. The third instruction depends on the results of the first two instructions. The third instruction needs the value of R1. The first instruction calculates R1 and stores it at the end of its S4 Stage. The third instruction fetches R1 in its S3 stage. By the time the third instruction needs R1, the first instruction has already completed, so there is no hazard for R1.

The third instruction needs the value of X, which is calculated by the second instruction. The third instruction will try to fetch the operand X in its S3 stage. At the same time, the second instruction is in its S4 stage executing and storing the new value for X. Because the new value for X is not yet stored when the third instruction needs it, a data hazard occurs. The third instruction will read the old value of X

26.

Three address:

ADD T1, B, C

ADD T2, D, E

MUL T1, T2

Two address:

MOVE T1, B

ADD T1, C

MOVE T2, D

ADD T2, E

MUL T1, T2

MOVE A, T1

One address:

LOAD B

ADD C

STORE T1

LOAD D

ADD E

MUL T1

STORE A

Zero address:

PUSH B

PUSH C

ADD

PUSH D

PUSH E

ADD

MUL

POP A