50.

X Y A | A_next B_next

0 0 0 |   0       1

0 0 1 |   1       1

0 1 0 |   0       0

0 1 1 |   0       1

1 0 0 |   1       1

1 0 1 |   1       1

1 1 0 |   1       0

1 1 1 |   0       1

For the JK flip-flop The inputs are J = X and K = Y' (because of the inverter). From JK flip-flop rules, If J = 0 and K = 0, the output A stays the same. If J = 0 and K = 1, it resets (A = 0). If J = 1 and K = 0, it sets (A = 1). If J = 1 and K = 1, it toggles.

For the D flip-flop, The D input is the output of the OR gate, which combines X and A (so D = X + A). The D flip-flop just transfers the D input to B at the next clock edge, so B_next = X + A.
The JK flip-flop’s behavior gives me A_next, and plugging X and A into the OR gate gives me B_next.

65.

a.

Input: 1 0 0 1 0 1 1 0

Start State: 00 (shift register: m1, m2 = 0,0)

Generator polynomials:

g1 = 111 (7 octal)

g2 = 101 (5 octal)

Calculation step-by-step:

Input=1, State=00 (0,0)

Register bits: [1, 0, 0]

Output1 = (1*1 + 0*1 + 0*1) mod 2 = 1

Output2 = (1*1 + 0*0 + 0*1) mod 2 = 1

Output = 11

New State = (1,0) = 10

Input=0, State=10 (1,0)

Register bits: [0, 1, 0]

Output1 = (0*1 + 1*1 + 0*1) mod 2 = 1

Output2 = (0*1 + 1*0 + 0*1) mod 2 = 0

Output = 10

New State = (0,1) = 01

Input=0, State=01 (0,1)

Register bits: [0, 0, 1]

Output1 = (0*1 + 0*1 + 1*1) mod 2 = 1

Output2 = (0*1 + 0*0 + 1*1) mod 2 = 1

Output = 11

New State = (0,0) = 00

Input=1, State=00 (0,0)

Register bits: [1, 0, 0]

Output1 = 1, Output2 = 1

Output = 11

New State = (1,0) = 10

Input=0, State=10 (1,0)

Register bits: [0, 1, 0]

Output1 = 1, Output2 = 0

Output = 10

New State = (0,1) = 01

Input=1, State=01 (0,1)

Register bits: [1, 0, 1]

Output1 = (1*1 + 0*1 + 1*1) mod 2 = 0

Output2 = (1*1 + 0*0 + 1*1) mod 2 = 0

Output = 00

New State = (1,0) = 10

Input=1, State=10 (1,0)

Register bits: [1, 1, 0]

Output1 = (1*1 + 1*1 + 0*1) mod 2 = 0

Output2 = (1*1 + 1*0 + 0*1) mod 2 = 1

Output = 01

New State = (1,1) = 11

Input=0, State=11 (1,1)

Register bits: [0, 1, 1]

Output1 = (0*1 + 1*1 + 1*1) mod 2 = 0

Output2 = (0*1 + 1*0 + 1*1) mod 2 = 1

Output = 01

New State = (0,1) = 01

Output string (grouped): 11 10 11 11 10 00 01 01

Output string (continuous): 1110111110000101

Answer for (a): 1110111110000101

b) In which state is the encoder after the sequence in part a is read?

From the final step above, after processing the 8th input bit, the new state is 01.

Answer for (b): 01

c) Which bit is in error in the string, 11 01 10 11 11 11 10? What is the probable value of the string?

Using the Viterbi algorithm (trellis computation done on scratch paper), the most likely transmitted sequence corresponds to the input bits: 1 1 0 0 1 1 0.

Re-encoding this input sequence from state 00 gives the correct encoded output:

Input: 1, State=00 -> Output=11, Next State=10

Input: 1, State=10 -> Output=01, Next State=11

Input: 0, State=11 -> Output=01, Next State=01

Input: 0, State=01 -> Output=11, Next State=00

Input: 1, State=00 -> Output=11, Next State=10

Input: 1, State=10 -> Output=01, Next State=11

Input: 0, State=11 -> Output=01, Next State=01

Correct encoded string: 11 01 01 11 11 01 01

Comparing with the received string: 11 01 10 11 11 11 10

The first discrepancy is in the 3rd pair. The first bit of this pair is different.

Counting the bits from the start of the sequence, the 5th bit is the one in error.

Probable value of the string is the corrected one: 11 01 01 11 11 01 01

Answer for (c):

The bit in error is the 5th bit of the received sequence.

The probable value of the string is 11010111110101.