10

a) Total bits needed = 2M * 16 = 32 Mbits
Size of one chip = 256K * 8 = 2 Mbits
Number of chips = (32 Mbits) / (2 Mbits) = 16 chips.

b) A full word is 16 bits. Each chip gives 8 bits. So, 16 / 8 = 2 chips are needed to get a full word.

c) A chip has 256K addresses.
256K = 2^8 * 2^10 = 2^18.
18 address bits are needed for each chip.

d) Number of banks = Total memory words / Words per chip
= 2M / 256K = 8 banks.

e) The total memory has 2M words.
2M = 2 * 2^20 = 2^21.
21 address bits are needed for the whole memory.

f) For high-order interleaving, the most significant bits select the bank.
Total address bits = 21
Bits for offset in chip = 18
Bits for bank select = 21 - 18 = 3
The address is 14 (decimal), which is E in hex. In 21-bit binary, it's 000 00000000000001110.
The top 3 bits are 000, so it is in Bank 0.
The offset is the remaining 18 bits, which is 14/E.
Location: Bank 0, Address E.

g) For low-order interleaving, the least significant bits select the bank.
The address is 14 (decimal) or 00000000000000001 110 in binary.
The bottom 3 bits are 110, which is 6 in decimal. So it's in Bank 6.
The offset is the remaining top 18 bits, which is 00000000000000001, or 1.
Location: Bank 6, Address 1.

18.

E

Total Address Bits: The total memory is 2048 bytes. To find the number of bits needed to address the whole memory, we need to solve 2^x = 2048. Since 2^11 = 2048, we need a total of 11 address bits.

Address on Chip: Each chip is 64 x 8, which means it has 64 different memory locations. To find the bits needed to address these locations, we solve 2^y = 64. Since 2^6 = 64, we need 6 bits for the address on the chip.

Chip Select Bits: The remaining bits are used to select which chip we are talking to.

Total bits - Address on chip bits = Chip select bits

11 - 6 = 5 bits.

24.

a)

100: 1108
101: 3109
102: 210B
103: A000
104: 6000
105: B10B
106: 2109
107: 7000

b)

| Label | Address |
| --- | --- |
| Start | 100 |
| A | 108 |
| B | 109 |
| C | 10A |
| D | 10B |

c)

- LOAD A: AC = 00FC

- ADD B: AC = 00FC + 000E. AC = 010A

- STORE D: D = 010A

- CLEAR: AC = 0

- OUTPUT: 0

- ADDI D: The address of D is 10B. The value stored at address 10B is 010A. So, we go to address 010A to get the data to add. The value at address 10A is C, which is 0108. AC = AC + 0108. AC = 0 + 0108. AC = 0108.

- STORE B: B = 0108.

- HALT: The program stops. AC = 0108.

AC = 0108.

25.

a)

200: 1209
201: 320A
202: 420B
203: 220C
204: 8800
205: 9208
206: C20C
207: A000
208: 7000

b)

| Label | Address |
| --- | --- |
| Begin | 200 |
| Loop | 202 |
| Done | 208 |
| Base | 209 |
| Offs | 20A |
| One | 20B |
| Addr | 20C |

c)

Here's the step-by-step:

- LOAD Base: AC = 200

- ADD Offs: AC = 200 + 9 = 209

- SUBT One: AC = 209 - 1 = 208

- STORE Addr: Addr (at memory location 20C) = 208.

- SKIPCOND 800:  AC is 208, which is greater than 0. So, we skip the next instruction.

- JUMP Done: This instruction is skipped.

- JUMPI Addr: This instruction jumps to the address stored in Addr. We just stored 208 in Addr. So, the program jumps to address 208.

- HALT: The program stops. The AC still contains the value from step 3, which is 0208.

AC = 0208.